# Table of Contents

Table of Figures	1

1. Design Explanation – Smart Water Gate System	2

Key Parts & Their Functions:	2

Gate in Closed Position:	4

Gate in Open Position:	4

2. Design Explanation - Pot-Based Paddy Irrigation Prototype	5

Components & Their Functions	5

3. Integrated System Design: Water Gate to Pot-Based Irrigation	6

Description of the Integrated System:	6

Predictive Model for Water Requirement:	7

System Behavior:	7

Key Integration Points:	7

# Table of Figures

Figure 1: Water tank with water gate	2

Figure 2: Gate in closed position	4

Figure 3: Gate in open position	4

Figure 4: Pot based paddy irrigation	5

Figure 5: Water gate to pot based irrigation	6

# 1. Design Explanation – Smart Water Gate System

Overview:

The system is a vertically sliding water gate used to regulate water flow in paddy fields. It uses a NEMA 17 stepper motor connected to a lead screw, which lifts or lowers the gate panel. The design ensures precision, automation, and protection against over-irrigation or water shortage.

## Key Parts & Their Functions:

Figure 1: Water tank with water gate

 Water Tank / Canal

- Purpose: Holds or channels the irrigation water.

- Description: This is the section from which water is either held back or released depending on the gate position.

- Placement: Below and behind the gate door.

 Gate Panel (Sliding Door)

- Purpose: Acts as a barrier that controls water flow.

- Material Suggestion: Stainless steel, acrylic, or treated wood (to resist corrosion and pressure).

- Movement: Moves vertically up to open and down to close.

 Lead Screw

- Purpose: Converts the motor’s rotational motion into linear motion to raise/lower the gate.

- Type: TR8x8 or T8 threaded rod.

- Mounted Between: Stepper motor (top) and gate nut (fixed to panel).

 Lead Screw Nut

- Purpose: Attached to the gate; travels along the screw when it rotates.

- Note: This nut moves up/down while preventing rotation. It’s critical that it’s fixed to the gate and constrained against rotation.

 NEMA 17 Stepper Motor

- Purpose: Provides precise rotational motion to the lead screw.

- Control: Driven by Raspberry Pi (through a driver like TMC2209 or A4988).

- Placement: Top of the gate frame.

Water Level Sensor

- Purpose: Measures the current water level in the tank or canal.

- Use: Helps determine whether the gate should be opened or closed based on tank water availability.

- Type Suggestions: Ultrasonic sensor (e.g., HC-SR04), float switch, or capacitive level sensor.

- Placement: Mounted inside or above the tank/canal facing the water surface.

## Gate in Closed Position:

Figure 2: Gate in closed position

In the closed state, the gate is fully lowered into the water channel, blocking the flow of water completely.

## Gate in Open Position:

Figure 3: Gate in open position

In the open state, the gate is lifted vertically by the motor, allowing uninterrupted water flow through the channel.

# 2. Design Explanation - Pot-Based Paddy Irrigation Prototype

Objective:

This setup is used to simulate a real-world paddy field scenario in a controlled environment using a plastic pot. It helps test the functionality and responsiveness of the smart water gate system by monitoring moisture levels and rain detection.

## Components & Their Functions

Figure 4: Pot based paddy irrigation

- Plastic Pot

- Purpose: Serves as a miniature paddy field for testing.

- Contents: Filled with soil and paddy plants.

- Advantage: Portable, safe for sensor trials, and easy to monitor in indoor/outdoor conditions.

- Paddy Plants

  - Purpose: Represent the crop being cultivated.

  - Significance: Soil moisture and water level are monitored to ensure the ideal growing environment.

- Soil Moisture Sensor

  - Placement: Inserted into the soil inside the plastic pot.

  - Purpose: Measures real-time soil moisture content.

  - Output: Analog or digital values sent to the controller (Raspberry Pi or Arduino).

  - Use Case: Triggers irrigation when moisture falls below a defined threshold.

- Rain Sensor (YL-83)

  - Placement: Positioned next to the pot in an elevated and exposed angle.

  - Purpose: Detects the presence of rain.

  - Output: Sends a digital LOW signal when raindrops are detected.

  - Use Case: Prevents irrigation during rainfall to conserve water.

# 3. Integrated System Design: Water Gate to Pot-Based Irrigation

The following diagram represents the complete integrated setup combining the smart water gate system and the pot-based paddy irrigation unit. This configuration demonstrates a closed-loop intelligent irrigation system where decisions are made based on real-time sensor data and predictive modeling.

Figure 5: Water gate to pot based irrigation

## Description of the Integrated System:

- On the left side, the Smart Water Gate System is installed over a cylindrical water tank. It consists of a NEMA 17 stepper motor and a lead screw mechanism that lifts or lowers the gate vertically to regulate water flow.

- Inside the tank, a Water Level Sensor is positioned to monitor the available water. This data is crucial for predicting whether irrigation can proceed.

- When the gate is opened, water exits the tank via the Water Flow Path—a sloped channel or pipe that delivers water to the crop area.

- On the right side, the Pot-Based Paddy Irrigation Unit is positioned to receive this water. It includes:

  - A Soil Moisture Sensor embedded in the soil to detect real-time moisture levels.

  - A Rain Sensor placed nearby to detect rainfall events.

  - A plastic pot filled with soil and planted with paddy to simulate field conditions.

## Predictive Model for Water Requirement:

To enhance automation, a machine learning model is used to predict the current water requirement for the paddy crop. The model takes the following inputs:

- Soil Moisture Level

- Water Level in the Tank

- Other (temperature, future rainfall etc.)

Based on these inputs, the model outputs whether irrigation is currently needed. This decision is then used to control the gate mechanism.

## System Behavior:

| Condition | System Response |
| --- | --- |
| Model predicts water is needed and no rain detected | Gate opens, water flows to the pot |
| Soil is moist or rain is detected | Gate remains closed |
| The tank water level is too low | Gate remains closed, irrigation blocked |

- The gate closes automatically once the moisture level is sufficient or if rainfall is detected.

- The system ensures minimal water usage by combining sensor readings with model-based prediction.

## Key Integration Points:

| Component | Connected To / Function |
| --- | --- |
| Smart Water Gate | Controls the physical release of water from the tank |
| Water Level Sensor | Sends real-time tank level data to the prediction model |
| Soil Moisture Sensor | Monitors current soil dryness and sends data to the model |
| Rain Sensor | Prevents water gate operation during natural rainfall |
| Water Flow Path | Transfers water from the tank directly to the paddy pot |
| Predictive Model | Computes whether irrigation is required (based on sensor input) |
| Paddy Pot | Final recipient of the irrigated water |