## Research Paper 1 - Estimation of Soil Moisture in Paddy Field Using Artificial Neural Networks

Original Research Paper Link - https://www.researchgate.net/publication/235891972_Estimation_of_soil_moisture_in_paddy_field_using_Artificial_Neural_Networks

My Google Drive Link - https://drive.google.com/file/d/14nqXHfXogtkoq4K1T4EqZiXIMT7odQoK/view?usp=drive_link

Summary of the Paper

  Authors: Chusnul Arif et al.

  Title: Estimation of Soil Moisture in Paddy Field Using Artificial Neural Networks

  Approach: The paper proposes two ANN models:

- To estimate reference evapotranspiration (ETo) using temperature inputs.

- To estimate soil moisture using ETo, precipitation, and crop coefficient.

  Data Limitations: The study focuses on limited meteorological data due to cost and accessibility challenges.

  Validation: Model achieved R² = 0.80 (training), R² = 0.73 (validation) using historical datasets.

Identified Technological Gaps

| Area | Existing Paper | Your Proposed System | Technological Gap |
| --- | --- | --- | --- |
| Data Sources | Uses only temperature and precipitation data for soil moisture prediction. | Uses real-time sensor data (soil moisture, tank water level) and weather forecasts (rain prediction). | Lack of real-time sensor integration and proactive prediction using weather forecasts. |
| Control Mechanism | No implementation of irrigation control—only prediction. | Automates irrigation by opening a water gate using motorized control based on predictions. | Absence of automated actuation mechanism for irrigation in the paper. |
| Scope | Focused on soil moisture estimation only. | Combines prediction with action (decision-making + irrigation execution). | Paper does not close the loop from estimation to automated water delivery. |
| Intelligence | Uses simple feedforward ANN with limited inputs and fixed parameters. | Can leverage advanced AI models (like RNNs, LSTMs, or hybrid models) to learn from temporal patterns and forecast more accurately. | Lacks exploration of temporal learning models and AI adaptation over time. |
| Infrastructure | Requires human effort to interpret model outputs and act accordingly. | Designed for autonomous system with minimal human intervention. | Gap in building an end-to-end autonomous solution. |

Value Addition in my FYP

My Final Year Project extends the capabilities of existing research in these ways:

- Integrates sensor networks for real-time environmental monitoring.

- Uses predictive modeling + future weather forecasts to adjust irrigation needs dynamically.

- Introduces motorized water gate control for automatic irrigation—making it a closed-loop smart farming solution.

- Can improve water use efficiency in paddy cultivation, especially relevant for regions with water scarcity.

## Research Paper 2 - Generating Soil Salinity, Soil Moisture, Soil pH from Satellite Imagery and Its Analysis

Original Research Paper Link - https://www.researchgate.net/publication/335358334_Generating_Soil_Salinity_Soil_Moisture_Soil_pH_from_Satellite_Imagery_and_Its_Analysis

My Google Drive Link - https://drive.google.com/file/d/1N5coe3s8d49MikTWiKb2n46sAZFQQrEs/view?usp=drive_link

Summary of the Research Paper

- Authors: Ghazali et al.

- Title: Generating Soil Salinity, Soil Moisture, Soil pH from Satellite Imagery and Its Analysis

- Approach: Uses Landsat 8 satellite imagery combined with lab/field data to estimate:

  - Soil moisture (SMI),

  - Soil salinity (SSI),

  - Soil pH (SpHI)

- Models Used: Regression-based “paddy leaf” and “bare soil” models

- Phenological Consideration: Soil parameters monitored across pre-harvest and post-harvest phases.

- Accuracy: Achieved RMSE = 1.40 and PRMSE = 24%.

Identified Technological Gaps

| Area | Satellite-Based Model (Ghazali et al.) | Your Proposed Smart Gate System | Technological Gap |
| --- | --- | --- | --- |
| Data Source | Relies on multi-temporal satellite imagery and field survey data | Uses real-time on-ground sensors (moisture, water level, weather forecast) | Satellite imagery is not real-time, limited by revisit cycles and cloud cover |
| Irrigation Control | Purely observational and analytical—no control action | Introduces motorized gate control that responds to predictions | No feedback/actuation mechanism exists in the paper |
| Granularity | Resolution is coarse (30m² per pixel) | Targets local, fine-grained control using ground-level sensors | Remote sensing can't support plot-level decision-making reliably |
| Prediction Use Case | Uses SMI/SSI/SpHI for monitoring and mapping | Uses sensor + forecast data to predict and act on future needs | Gap in integrating predictive irrigation decision logic |
| Adaptability | Static regression models, not real-time adaptable | Capable of learning continuously and adjusting dynamically (e.g., LSTM models) | Models are not adaptive to dynamic field conditions |

### Value Addition in my Smart Water Gate FYP

My system improves on the satellite-based monitoring approach by:

- Enabling real-time control through physical gate actuation.

- Utilizing fine-resolution sensor data (soil moisture, water level).

- Including future weather conditions (rain prediction) into the logic.

- Designing an end-to-end closed-loop solution: monitor → predict → act.

## Research Paper 3 - IoT based Smart Irrigation System using Machine Learning Models

Original Research Paper Link - https://www.sciencedirect.com/science/article/pii/S0957417422009563

My Google Drive Link - https://drive.google.com/file/d/1wzw1K3nR9RBZgW3HETRYY3cfkxsddc4n/view?usp=drive_link

### Summary of the Research Paper

- Authors: P. Raja et al.

- Title: IoT based Smart Irrigation System using Machine Learning Models

- Published In: Expert Systems with Applications, Elsevier (2022)

- Core Features:

  - Combines IoT devices (sensors for soil moisture, humidity, temperature).

  - Employs machine learning models (Random Forest, SVM, Naive Bayes, XGBoost) to predict irrigation needs.

  - Uses weather APIs to include environmental context.

  - Sends irrigation decisions to a relay circuit (solenoid valve for irrigation).

  - Achieves 95.6% prediction accuracy with RF.

Identified Technological Gaps

| Area | Existing IoT System (Raja et al.) | Your Proposed System | Technological Gap |
| --- | --- | --- | --- |
| ML Models | Uses general-purpose ML models (RF, SVM, etc.) for classification | Can explore temporal models (LSTM/RNN) or hybrid decision logic for better time-series irrigation prediction | Existing models may lack temporal insight needed for dynamic soil conditions |
| Control | Uses relay switch to control solenoid valves | Uses motorized water gate mechanism—potentially more robust for paddy field irrigation | Paper focuses on pipe irrigation, not suitable for field-level water gate systems |
| Environmental Inputs | Includes weather forecasts, temperature, humidity, and soil moisture | Adds tank water level sensor and focuses specifically on rain prediction and water availability | Lacks water source feedback (e.g., tank monitoring) in the irrigation loop |
| Crops and Context | General applicability, no specific tuning for paddy fields | Tailored for paddy cultivation, which has unique waterlogging and drainage needs | No crop-specific logic or constraints modeled in the referenced system |
| Automation Design | Basic control via relay, focuses mostly on decision making | Full integration of control and mechanical automation (gate operation, water release control) | Your system introduces electro-mechanical integration, which is missing in the paper |
| Adaptability & Customization | No adaptive control logic across growing stages or changing water availability | Includes potential to adapt to crop growth stages, forecast changes, and tank levels | Static rule application; lacks adaptive irrigation policy formulation |

### Value Addition in my Smart Water Gate FYP

My project advances the state-of-the-art by:

- Implementing smart gate actuation for paddy-specific needs rather than generic irrigation valves.

- Considering water tank levels, making it more sustainable and resource-conscious.

- Using weather forecast (especially rain prediction) for proactive irrigation control.

- Opening opportunities for long-term deployment with learning-based control systems (e.g., reinforcement learning or LSTM-based predictions).

- Focusing on real-time irrigation for paddy, which requires more nuanced water handling compared to general horticulture.

## Research Paper 4 - Estimating Soil Moisture Condition of Paddy Fields by Using Optical Remote Sensing Imagery

Original Research Paper Link -

https://iopscience.iop.org/article/10.1088/1755-1315/1109/1/012067

My Google Drive Link - https://drive.google.com/file/d/1zjxAclg9nUwY0GesJE7oSeI2rmLCraIB/view?usp=drive_link

### Summary of the Research Paper

- Authors: Rizqi I’anatus Sholihah et al.

- Conference: IOP Conf. Ser.: Earth and Environmental Science, 2022

- Focus: Used Landsat 8 OLI/TIRS data to estimate:

  - Soil Moisture Index (SMI)

  - Vegetation Index (NDVI)

  - Land Surface Temperature (LST)

- Methodology:

  - Derived LST from thermal infrared data.

  - Used NDVI and LST to calculate SMI for paddy fields in East Java, Indonesia.

  - Mapped spatial distribution of soil moisture using satellite imagery.

Identified Technological Gaps

| Area | Remote Sensing Model (Sholihah et al.) | Your Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Data Type | Optical satellite imagery (Landsat 8) | Real-time ground sensors (moisture, tank level, temperature forecast) | Lack of real-time and on-field granularity in the paper |
| Purpose | Soil moisture monitoring and mapping | Predictive model to act and irrigate accordingly | Monitoring only; lacks automated response system |
| Spatial Resolution | Medium spatial resolution (30m), passive sensing | Local plot-level control with precise irrigation via gate control | Can't be used for micro-level irrigation decisions |
| Environmental Integration | Uses NDVI and LST as indirect proxies | Includes direct sensing + future rain prediction for accurate irrigation timing | No weather or climate forecast integration |
| Temporal Frequency | Periodic imagery based on satellite overpass (8–16 days) | Near real-time decision-making based on sensor input | Temporal gap leads to delayed insight, not suitable for dynamic control |
| Irrigation Feedback | No decision-making or control systems | Fully integrates automated gate with motorized control | No feedback loop from monitoring to execution in the paper |

### Value Addition in my Smart Water Gate FYP

My system improves on satellite-based monitoring by:

- Closing the loop from prediction to real-world irrigation control.

- Real-time adaptability, responding to immediate field conditions(moisture and tank level sensors) and forecasts.

- Using fine-grained data from on-site sensors suitable for paddy fields.

- Enabling automated control of irrigation infrastructure with motorized gates.

- Tailoring for paddy-specific requirements, which satellite imagery cannot address directly.

## Research Paper 5 - A low-cost approach for soil moisture prediction using edge computing devices and machine learning

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S0048969722021593

My Google Drive Link –

https://drive.google.com/file/d/1DdFd1Kwa8rFD2hmeK-T5B86jbc-p9vwx/view?usp=drive_link

### Summary of the Research Paper

- Authors: G. Ramesh Kumar et al.

- Published In: Science of the Total Environment, 2022

- Objective: Design a low-cost, edge-based solution for soil moisture prediction.

- Method:

  - Utilizes on-field sensors for soil moisture, temperature, humidity, etc.

  - Performs prediction using ML models (Random Forest, ANN) directly on edge devices (e.g., Raspberry Pi).

  - Focused on reducing latency and dependency on cloud systems.

- Key Contributions:

  - Real-time inference on edge devices.

  - Promotes cost-effectiveness for small-scale farming.

  - Achieves good accuracy with ML models, emphasizing affordability and independence.

Identified Technological Gaps

| Area | Edge-Based Prediction (Ramesh Kumar et al.) | Your Proposed System | Technological Gap |
| --- | --- | --- | --- |
| Prediction Platform | Uses ML models deployed on edge devices | Could adopt AI models that also consider forecast and real-time irrigation control | Focuses on prediction only, not on integration with mechanical irrigation |
| Control Mechanism | No control element—no irrigation action triggered | Introduces motorized water gate that opens automatically | Missing the actuation part of the feedback loop |
| Environmental Context | Uses sensor data like soil temperature and humidity | Adds tank water level and rain prediction, making it more context-aware | Rain forecast & water availability not integrated in their model |
| System Scope | Soil moisture prediction only; assumes human intervention post-prediction | Fully autonomous irrigation system that requires no human input post-deployment | No end-to-end automation in the proposed system |
| Target Crop | General agriculture (not crop-specific) | Designed for paddy fields, which have unique water needs (flooding, drainage) | Lacks crop-specific optimization, which is key in your FYP |
| Hardware Focus | Focuses on affordable ML deployment via edge | Also includes electromechanical integration for water control | Paper does not address hardware actuation or water infrastructure management |

### Value Addition in My Smart Water Gate FYP

My system takes the core idea of edge-based prediction further by:

- Integrating mechanical actuation (smart gate control), turning prediction into action.

- Including weather forecast and water tank level monitoring to optimize water use.

- Designing for paddy-specific scenarios, addressing irrigation peculiarities like field flooding.

- Maintaining autonomous operations with little to no farmer intervention.

- Potentially blending on-device inference with cloud support for long-term learning.

## Research Paper 6 - Soil moisture quantity prediction using optimized machine learning algorithm

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S2210537918302464

My Google Drive Link - https://drive.google.com/file/d/1uXyVJdG8tWGFabK3n3-FzW12v5RYIQu7/view?usp=drive_link

### Summary of the Research Paper

- Authors: Nirala et al.

- Published In: Sustainable Computing: Informatics and Systems (2020)

- Focus: Predicting quantitative soil moisture levels using optimized ML techniques.

- Methodology:

  - Applies several ML models: SVM, ANN, Random Forest, and ensemble classifiers.

  - Uses optimization techniques (e.g., firefly algorithm) to tune model parameters.

  - Data collected from weather stations and field sensors (temperature, humidity, etc.).

- Goal: Improve accuracy and generalization of moisture prediction systems for precision agriculture.

- Result: ANN with Firefly optimization yielded the best performance (98% accuracy).

Identified Technological Gaps

| Area | Optimized ML Approach (Nirala et al.) | Your Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Purpose | Focuses on improving prediction accuracy through algorithmic tuning | Focuses on closing the loop from prediction to actuation | Paper does not progress to real-world irrigation actions |
| Environmental Inputs | Uses standard sensor inputs (humidity, temperature) | Incorporates rain prediction and tank water levels for proactive irrigation | No forecasting or water availability logic present |
| Control Element | Predicts soil moisture; assumes external action by farmers | Automates the control of a water gate using motor mechanisms | No hardware integration or field deployment planning |
| System Autonomy | Semi-automated; still requires manual action post-prediction | Fully autonomous—runs end-to-end with minimal human input | No transition from smart analytics to smart action in their system |
| Crop-Specific Focus | General application; not specific to paddy or waterlogged fields | Designed for paddy field conditions, considering unique water requirements | Lacks crop-specific customization or adaptation logic |
| Temporal Dynamics | Models trained on static datasets | Can integrate time-series forecasting using models like LSTM | Lacks temporal learning and adaptation to field changes |

### Value Addition in My Smart Water Gate FYP

My project builds on this predictive work by:

- Applying real-time predictions to a motorized water gate for paddy-specific irrigation.

- Leveraging additional real-world variables like tank level and rain forecasts to improve decision-making.

- Enabling temporal forecasting to adjust irrigation based on weather trends and crop stages.

- Closing the AI–mechanical action loop, offering a practical solution for water conservation.

- Designing for paddy-specific scenarios, addressing irrigation peculiarities like field flooding.

## Research Paper 7 - Soil moisture quantity prediction using optimized machine learning algorithm

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S0016706117321031

My Google Drive Link - https://drive.google.com/file/d/1huQcC0gNh5N65YFNx3Ds4FTrvNGJG4fF/view?usp=drive_link

### Summary of the Research Paper

- Authors: Shahaboddin Shamshirband et al.

- Published In: Geoderma, 2018

- Focus: Short-term soil moisture forecasting using a hybrid ML model.

- Technique:

  - Combines M5 model tree (a rule-based regression model) with the firefly optimization algorithm.

  - Uses meteorological inputs: rainfall, temperature, evaporation, humidity.

  - Forecasts soil moisture over short horizons with high accuracy.

- Performance: Achieved RMSE and MAE values superior to standalone models (e.g., ANN, RF, SVM).

Identified Technological Gaps

| Area | Hybrid Forecasting Model (Shamshirband et al.) | Your Proposed System | Technological Gap |
| --- | --- | --- | --- |
| Prediction Technique | Uses hybrid ML (M5 + firefly) for time-series soil moisture forecasting | Can use LSTM, ensemble or RL-based models for real-time decision-making | No neural or adaptive learning for irrigation control |
| Actuation Layer | Purely predictive—no control or automation component | Uses motorized gate to act on predictions and control irrigation | Missing physical irrigation implementation entirely |
| Input Variables | Rainfall, temperature, humidity, evaporation | Adds tank water level and real-time soil moisture sensors | No integration of water source conditions or sensor fusion |
| System Output | Provides numerical soil moisture forecasts | Uses predictions to automatically manage irrigation | Forecast output not connected to practical execution |
| Field Realism | Model tested on existing weather station data | Designed for real deployment in paddy fields, including rainwater management | No field integration strategy mentioned |
| Time-Awareness | Forecasts short-term conditions (24–48h) | Adjusts dynamically based on live weather updates + future rain prediction | Model lacks forecast update adaptability during real-time deployment |

### Value Addition in My Smart Water Gate FYP

My system:

- Applies predictions directly into practical irrigation systems (gate-based control).

- Fuses real-time sensors (e.g.- tank level) with forecasting data (e.g.-rain forecast), enabling intelligent decision-making.

- Goes beyond numerical modeling by delivering an end-to-end automated solution for paddy irrigation.

- Supports fine-grained control tailored to paddy field requirements, not just generic soil moisture estimation.

## Research Paper 8 - Deep learning for proximal soil sensor development: A survey

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S095741742200269X

My Google Drive Link - https://drive.google.com/file/d/1EyppEiKk86mwOdT1DlyenwjcFidrcDJg/view?usp=drive_link

### Summary of the Research Paper

- Authors: Zhang et al.

- Published In: Expert Systems with Applications, 2022

- Purpose: Survey of deep learning applications for proximal soil sensing.

- Scope:

  - Focuses on developing and optimizing soil sensors using DL.

  - Covers applications like soil moisture, salinity, nitrogen detection via:

    - Spectral data (e.g., NIR)

    - Imaging

    - Sensor fusion techniques

  - Discusses deep networks: CNNs, autoencoders, LSTM for sensor calibration and improvement.

- Key Insight: DL enhances sensor data interpretation, model accuracy, and calibration—but systems are largely diagnostic, not autonomous.

Identified Technological Gaps

| Area | DL-Based Sensor Development (Zhang et al.) | Your Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Main Focus | Improve sensor precision using deep learning (via RGB image classification) | Build a smart irrigation system using real-time multi-sensor inputs and predictions | Focuses on sensor development and classification, not predictive decision-making or control |
| System Design | Concerned with data classification using visual features (image textures) | Incorporates AI prediction + mechanical actuation (motorized water gate) | No integration with physical irrigation hardware or multi-sensor decision systems |
| Control Layer | No mention of gate control, irrigation infrastructure, or field execution | Automates water flow via real-time logic and physical gate actuation | No automation or control pipeline to close the feedback loop |
| Prediction Usage | DL improves soil moisture class recognition (classification, not forecasting) | Uses ML to predict future irrigation needs (forecasting + regression models) | No time-based forecasting or future moisture requirement prediction is done |
| Deployment Strategy | Built for controlled conditions (static camera + pot soil images); not field-tested | Field-ready, using low-cost sensors, controllers, and adaptive prediction models | Not validated in dynamic, real-world field environments |
| Real-time Automation | Only offline training and classification; no real-time control system discussed | Designed for autonomous, closed-loop real-time irrigation | No real-time control or feedback — remains a sensing system only |
| Environmental Adaptability | Limited to fixed lighting and fixed soil pot setups | Built to work across changing field, weather, and soil conditions | No demonstration of adaptability to field lighting, weather variability, or multiple crops |
| Scalability | Model limited to image input from a few soil types | Flexible system with scope for multi-sensor scaling and farm-wide deployment | No plan or method for scaling the model across larger or multiple fields |

Value Addition in My Smart Water Gate FYP

My system builds upon this sensing research by:

- Using multiple sensor inputs (soil, tank level, rain forecast) for real-world irrigation decision-making instead of Camera-based vision sensors.

- Connecting AI-powered analysis directly to field actuation (e.g., opening the gate).

- Enabling resource-aware, rain-forecast-based irrigation tailored to paddy field constraints.

- Designing an operational system that is deployable, not just experimental.

## Research Paper 9 - Estimation of root-zone soil moisture from ground and remote sensing data using machine learning

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S0034425721001528

My Google Drive Link - https://drive.google.com/file/d/12A9TAk2mSuKDg6rPfKsNdoTjdwTFAC75/view?usp=drive_link

### Summary

- Authors: Babaeian et al.

- Published In: Remote Sensing of Environment, 2021

- Focus:

  - Estimating Root Zone Soil Moisture (RZSM) using:

    - High-resolution UAV multispectral imagery (VIS, NIR)

    - Soil physical and hydraulic properties

    - Automated Machine Learning (AutoML) with multiple models (NN, DRF, GBM, GLM)

- Methodology:

  - Created 12 hierarchical ML models combining vegetation indices (NDVI, NTR) and soil data (texture, bulk density, porosity, field capacity, etc.)

  - Evaluated accuracy using TDR sensor ground-truth data at 2, 10, and 50 cm depths.

- Results:

  - Best RZSM performance when near-surface soil moisture was included in models.

  - RMSE < 0.02 cm³/cm³ and R² > 0.90 for best-performing models.

  - NDVI alone was insufficient; soil attributes were essential for deeper prediction accuracy.

- Key Contribution:

  - Demonstrated the power of AutoML combined with soil-sensor fusion for accurate field-scale moisture mapping.

Identified Technological Gaps

| Area | Root-Zone ML Estimation (Srivastava et al.) | Your Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Estimation Target | Focused on root-zone soil moisture prediction | Uses surface moisture + forecast + water availability for irrigation decision-making | Model limited to prediction only, not irrigation execution |
| Control Mechanism | No irrigation or actuation system considered | Automates water gate control using motorized mechanism | No physical integration or feedback control |
| Data Source | Remote sensing + ground-based for model training | Live sensor data + rain forecast + tank water level | Model trained on past data, not designed for real-time adaptation |
| Prediction Purpose | Improve RZSM estimation accuracy for monitoring | Predict when and how much to irrigate, then act | Focuses on insight only, not operational use |
| Sensor System | Relies on external satellite + sparse ground sensors | Uses in-field sensors for real-time feedback | Satellite data limits temporal and spatial resolution |
| Crop Specificity | General agricultural use | Designed specifically for paddy fields | No tailoring to flood-irrigation crops like paddy |

### Value Addition in My Smart Water Gate FYP

My project distinguishes itself by:

- Integrating real-time sensing and actuation, not just estimation.

- Being designed for paddy fields, which require different irrigation strategies than root-zone-based crops.

- Creating a feedback-enabled closed-loop system from prediction to gate control.

- Incorporating forecasting and water source status (tank level), which this paper does not consider.

## Research Paper 10 - Multi-layer high-resolution soil moisture estimation using ensemble learning and spatiotemporal fusion

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S0034425721004260

My Google Drive Link –

https://drive.google.com/file/d/1DjNO-sZZRH54MCGd9xMMJxy-akQRL5It/view?usp=drive_link

### Summary

- Authors: Srivastava et al.

- Published In: Remote Sensing, 2021

- Focus:

  - Predicting Root-Zone Soil Moisture (RZSM) using:

    - Surface Soil Moisture (SSM) data from ESA-CCI and SMAP satellites

    - Vegetation indices (NDVI)

    - Meteorological data

    - Machine Learning models (Random Forest, Support Vector Regression, Artificial Neural Network)

- Goal:

  - Bridge the gap between surface and root-zone soil moisture using hybrid data-driven modeling.

- Methodology:

  - Trained ML models on SSM + auxiliary inputs to predict soil moisture at multiple depths (0–100 cm).

  - Model performance validated against in situ soil moisture data from SCAN and USCRN networks.

  - Used feature importance ranking to determine key predictors (NDVI, rainfall, air temperature).

- Results:

  - Random Forest (RF) consistently outperformed SVR and ANN.

  - Achieved R² > 0.80 for 0–100 cm depth predictions.

  - High-resolution maps (36 km SMAP data) were generated for operational forecasting.

Identified Technological Gaps

| Area | Multi-Layer Estimation (Wei et al.) | Your Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Focus | High-resolution multi-depth moisture estimation | Focused on surface-level control + rain + tank data for real-time irrigation | Paper prioritizes depth modeling, not irrigation use |
| Irrigation Control | No irrigation mechanism; only moisture mapping | Automates irrigation via motorized water gate | No integration with water distribution infrastructure |
| Data Integration | Uses advanced satellite + sensor fusion for spatial enhancement | Uses simple, local sensors + weather forecast for decision-making | Their system is data-heavy and requires external sources |
| Scalability | Research-grade system, dependent on satellite imagery | Scalable in-field system using low-cost sensors and open hardware | Approach may not be feasible for real-time control in small farms |
| Temporal Resolution | Improved using spatiotemporal fusion, but still satellite-constrained | Based on live sensor inputs and predictive models | Not truly real-time, thus limited for responsive irrigation |
| System Output | Soil maps and time-series analysis | Direct irrigation actions based on forecast and resource availability | No action layer—purely analytical outcome |

### Value Addition in My Smart Water Gate FYP

My system innovates beyond this work by:

- Delivering actionable outcomes—controlling irrigation gates, not just generating soil data.

- Relying on affordable, deployable tech—ideal for actual paddy farmers.

- Integrating real-time forecast + water availability, leading to sustainable irrigation.

- Focusing on surface-level irrigation logic, which is practical for paddy fields.

## Research Paper 11 - Automatic gate controlling system for sustainable agriculture water management in Sri Lanka: A review

Original Research Paper Link -

https://www.researchgate.net/publication/380288328_Automatic_gate_controlling_system_for_sustainable_agriculture_water_management_in_Sri_Lanka_A_review

My Google Drive Link –

https://drive.google.com/file/d/1JCvUnvZU-JnBYNNlSsC9A5uRu2qZjPo8/view?usp=drive_link

### Summary

- Authors: Rathnayaka et al.

- Published In: Precision Agriculture Science and Technology, 2024

- Focus:

  - Reviews and proposes a smart, fully automated irrigation system using:

    - Sensor nodes (for water level, soil moisture, temp, humidity)

    - Motorized sluice and feeder canal gates

    - Closed-loop PID controller

    - Web dashboard for remote monitoring

  - Designed for paddy irrigation in Sri Lanka’s dry zones to replace outdated, manual canal systems.

- System Highlights:

  - Sensor modes dynamically switch between regular and PID control.

  - Gate components: Reservoir gate, Main canal regulator, Feeder canal, Paddy field gate.

  - Wireless peer-to-peer ESP32 communication, solar-powered sensors, and cloud integration.

  - Emphasizes precision water distribution, real-time feedback, and emergency manual override.

- Conclusion:

  - Successfully demonstrates a closed-loop, locally deployable, scalable smart water gate system.

  - Addresses water scarcity, labor shortages, and sustainability.

  - One of the most complete real-world implementations of smart irrigation for paddy fields reviewed to date.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Machine Learning Use | Uses fixed PID and rule-based control logic | Integrates AI/ML models to predict required soil moisture and schedule irrigation | Lacks predictive intelligence for forecast-driven irrigation |
| Rain Forecasting | No rain prediction or environmental forecasting integrated | Uses weather APIs or ML-based rain predictors to optimize timing | No pre-irrigation decision making based on rainfall likelihood |
| Irrigation Requirement Estimation | Responds to sensor values (threshold-based) | Predicts how much water is required based on trends and forecasts | Only reactive; no proactive irrigation strategy |
| Crop Sensitivity or Scheduling | Single-mode water level strategy (e.g., 8 cm target) for all stages | Can adapt irrigation based on crop growth stage or evapotranspiration | Lacks dynamic scheduling for crop growth stages |

### Value Addition in My Smart Water Gate FYP

- Incorporates intelligent prediction models for soil moisture and rain, enabling proactive irrigation control rather than reactive gate triggering.

- Goes beyond PID logic by introducing adaptive AI-based decision-making to fine-tune water supply based on future conditions.

- Provides a modular and extendable architecture that supports crop-specific irrigation scheduling, improving yield and water efficiency.

- Maintains all practical advantages of the reviewed system (sensor nodes, PID control, solar power, remote dashboards) while enhancing with predictive capability and real-time optimization.

- Bridges the gap between smart sensing and smart reasoning, offering both automation and intelligence for sustainable irrigation.

## Research Paper 12 - Automated Water-Gate Controlling System for Paddy Fields

Original Research Paper Link -

https://www.researchgate.net/publication/349658847_Automated_Water-Gate_Controlling_System_for_Paddy_Fields

My Google Drive Link –

https://drive.google.com/file/d/1KcOSer-beg6FOGgovtEWghTIjnJ6ENap/view?usp=drive_link

### Summary

- Authors: Sanjula et al.

- Published In: IEEE ICAC, 2020

- Focus:

  - Proposes an IoT-based smart irrigation system to automate water distribution from reservoirs to paddy fields.

  - Uses sensor nodes, a paddy-field gate, and a reservoir gate, all coordinated through cloud-based control and a web dashboard.

  - Introduces two control modes:

    - Regular Mode: Maintains water at 8 cm based on sensor thresholds.

    - PID Mode: Uses closed-loop control to continuously maintain target water level.

  - Predicts short-term rain using a SVM model trained on Colombo weather data.

  - Includes ESP-NOW-based peer-to-peer networking, solar power, and cloud data logging.

- Implementation:

  - Real-time testing conducted with variable flow rates, temperature, and set-point maintenance under PID logic.

  - Achieved accurate 8 cm water level regulation using sensor feedback and gate height adjustment.

- Conclusion:

  - Offers a complete automation system from sensing to control for paddy irrigation using IoT and PID control. However, lacks intelligent prediction or optimization.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Decision Logic | Rule-based with thresholds and PID | AI-based predictions using ML to determine irrigation need | Lacks intelligent prediction model for soil moisture requirement |
| Forecast Integration | Short-term rain prediction using SVM from static dataset | Dynamic weather forecast using API or ML + sensor fusion | Forecasting model is static and location-limited |
| Moisture Requirement Estimation | Responds reactively to moisture thresholds | Predicts future water need and controls gate accordingly | No proactive prediction of irrigation demand |
| Adaptive Control | Fixed water level (8 cm) for all crop phases | Adaptive control based on crop growth stage, soil moisture trends | No support for dynamic scheduling based on crop or field variability |

### Value Addition in My Smart Water Gate FYP

- Introduces AI/ML-driven predictions for soil moisture and rainfall, enabling proactive irrigation decisions.

- Replaces static PID logic with adaptive models that account for crop phase, environment, and resource optimization.

- Provides a fully closed-loop system: from prediction → decision → action, linking sensors to automated gate control.

- Enhances forecast flexibility with real-time weather APIs or ML-based dynamic forecasting beyond fixed-location datasets.

- Designed for scalability and configurability, allowing tailored irrigation strategies for different fields or crops.

## Research Paper 13 - Design of Automatic Watergate System Using ESP32 Microcontroller Based on Fuzzy Logic Method

Original Research Paper Link -

https://ieeexplore.ieee.org/document/9678410

My Google Drive Link – https://drive.google.com/file/d/1CcrFYqL4cI7N9TvtHoSp861f6XeesBSL/view?usp=drive_link

### Summary

- Authors: Rama et al.

- Published In: IEEE, 2021

- Focus:

  - Designing a fuzzy logic-based automatic watergate system using:

    - ESP32 microcontroller

    - Ultrasonic water level sensor

    - Soil moisture sensor

    - DC motor-driven mechanical gate

  - The system determines the gate opening level (0%, 50%, or 100%) based on sensor input processed through a Mamdani fuzzy inference system.

- System Flow:

  - Sensor data (water level + soil moisture) → Fuzzy logic controller → Motorized gate actuation.

  - Radio modules (NRF24L01) used for wireless communication in remote environments.

  - MATLAB used for rule development and defuzzification logic testing.

  - Membership functions (low, normal, high) defined for both sensors; rules built for 9 possible input conditions.

  - Hardware tested with voltage calibration and signal propagation testing (with and without obstacles).

- Key Contributions:

  - Flexible gate control using fuzzy rules instead of binary thresholds.

  - Design adaptable to rural/remote areas with weak internet via radio-based communication.

  - Successfully simulated and tested with sensor accuracy and defuzzification validation.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Decision Logic | Fuzzy logic rules based on real-time water and soil moisture levels | ML/AI-based prediction of future irrigation requirement | No predictive modeling; reacts to present sensor data only |
| Forecast Integration | No use of weather or rain forecasts in decision making | Uses weather forecasts or ML rain predictors for planning irrigation | Cannot skip irrigation during upcoming rain events |
| Irrigation Scheduling | Limited to fixed output levels (0%, 50%, 100%) based on rule-based logic | Provides flexible irrigation control based on AI-estimated moisture demand | Less granular, no dynamic computation of irrigation quantity |
| Crop Specific Adaptation | Single logic rule set for any condition | Adaptable AI models that adjust based on crop stage and environmental changes | Fuzzy system cannot adapt to seasonal or crop-stage variation |

### Value Addition in My Smart Water Gate FYP

- Integrates ML-based soil moisture forecasting, offering proactive irrigation rather than reactive fuzzy thresholding.

- Combines fuzzy-style flexibility with AI adaptability, enabling system learning and evolution over time.

- Supports external data integration (weather APIs, forecast models) to prevent irrigation during forecasted rainfall.

- Fully modular and scalable; deployable with or without internet access using fallback communication like LoRa or offline logging.

- Optimizes water usage with real-time, AI-informed, dynamic actuation, beyond pre-defined fuzzy rule responses.

## Research Paper 14 - Gate Guard: Streamlining Agriculture with Smart Water Gate System

Original Research Paper Link -

https://ieeexplore.ieee.org/document/10841506

My Google Drive Link – https://drive.google.com/file/d/1WMRSkMOepjbzWnAwnLup_RRuWLQRcjjc/view?usp=drive_link

### Summary

- Authors: Niharika et al.

- Published In: Proceedings of IEEE ICACRS, 2024

- Focus:

  - Proposes an IoT-based water gate automation system for paddy fields using:

    - Arduino Uno controller

    - Water level sensor for detecting high/low field water levels

    - Servo motor for automated gate opening/closing

    - ESP8266 Wi-Fi module to send data to ThingSpeak cloud

    - GSM SIM800i module to send SMS alerts to farmers

- System Workflow:

  - Water level sensor triggers Arduino logic → If threshold is exceeded, gate opens → SMS sent to farmer and data logged to cloud.

  - 16x2 LCD shows real-time field water status.

  - Farmer receives alert: "WATER LEVEL HIGH, GATE OPENED".

  - System includes a web dashboard via ThingSpeak for real-time data access.

- Key Strengths:

  - Combines local sensor actuation with cloud-based remote monitoring.

  - Designed to work in rural, low-internet environments using GSM backup.

  - Minimizes human intervention and supports automated drainage to avoid waterlogging.

- Conclusion:

  - Presents a functionally complete low-cost smart gate system using basic sensors and microcontrollers.

  - However, it does not include predictive irrigation logic, AI-based adaptability, or advanced scheduling mechanisms.

Technological Gaps Compared to Your Proposed System

| Feature | Gate Guard (2024) | Your Proposed FYP | Technological Gap |
| --- | --- | --- | --- |
| Decision Logic | Basic rule-based thresholds | AI-based prediction of required soil moisture using multi-variable inputs | No intelligent model – no learning or forecasting |
| Environmental Inputs | Only current water level | Uses soil moisture, tank level, and weather forecast | Lacks multi-factor integration and predictive control |
| Adaptability | Static threshold; manually set | Adapts to field, season, and crop stage using AI | No self-calibration or context-awareness |
| Sensor Coverage | One sensor input (water level) | Combines multiple sensors: soil moisture, temp, water level | Limited environmental sensing |
| Forecast Integration | None | Uses weather and rain forecast to optimize timing | Cannot plan ahead for future conditions |
| Application Domain | General irrigation fields | Optimized for paddy cultivation, including flooding control | No paddy-specific modes or stages (e.g., transplanting) |
| Gate Control | Opens/closes gate based on one signal | Dynamically predicts when and how much to open | Less precision and no optimization model |
| Data Use | Cloud storage for logging only | Cloud/local model training, adaptive feedback | Data not used for continuous model improvement |

### Value Addition in My Smart Water Gate FYP

- Transitions from threshold-based to prediction-based control.

- Incorporates AI/ML models to determine when and how much water is needed.

- Integrates rain forecasts, soil data, and tank status to optimize decisions.

- Designed for field deployment with real-time, low-cost smart automation.

- Provides intelligent irrigation tailored for paddy crop cycles.

## Research Paper 15 - Design and Control System of Sluice Gate With Web-Based Information

Original Research Paper Link -

https://ieeexplore.ieee.org/document/9650409

My Google Drive Link – https://drive.google.com/file/d/15J8p0nHPRMsRF4wNB_I_RgAf6WoAy7-Z/view?usp=drive_link

### Summary

- Authors: Desnanjaya & Nugraha

- Published In: IEEE ICSGTEIS, 2021

- Focus:

  - Developed a web-based irrigation sluice gate system for flood-prone rain-fed rice fields in Bali.

  - Combines hardware (Arduino, ultrasonic sensor, servo motor) and software (SIM900, website dashboard).

  - Automatically monitors water level at the field gate (“Temuku”) and allows farmers to control it remotely via a web interface or SMS.

- System Components:

  - Arduino Uno – main processor

  - Ultrasonic HC-SR04 – water level sensor

  - SIM900 module – sends sensor data to database and receives SMS commands

  - Servo motor – controls gate opening/closing

  - LCD + LED – displays local water level status

- User Roles:

  - General users – view water levels

  - Pekaseh (subak leader) – manage accounts

  - Farmers – control sluice gates via dashboard (open/half-open/close)

- Testing & Results:

  - Successfully transmitted real-time water level to cloud.

  - Gate control via web was reliable (98% success) despite some SMS delays due to signal.

  - ISO 1926 usability metrics:

    - Functionality: ✅ 100%

    - Reliability: ✅ 99%

    - Usability: ✅ 66.75%

    - Efficiency: ✅ Avg. 650.8 ms load time

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Control Intelligence | Manual SMS/web-based gate control based on observed water level | AI/ML predicts future irrigation need based on multiple sensors | No predictive control or intelligent scheduling; system is fully reactive |
| Rain Forecasting | Not integrated; decisions made solely on current water level | Integrates weather forecast/rain prediction to avoid unnecessary irrigation | Lacks environmental data fusion for optimal water usage |
| Irrigation Need Estimation | Gate controlled when level exceeds 5 cm; no moisture or evapotranspiration logic | Predicts how much water is needed and when based on soil + weather + crop | No estimation of moisture requirement; cannot adapt to crop needs |
| Automation Level | Partially automated via SMS/web buttons | Fully autonomous system that closes loop from sensor → prediction → actuation | Still dependent on human input for triggering actions |
| Scalability | Designed for single village (Bali) | Designed for wider paddy deployments in Sri Lanka | Not scalable or auto-learning |

### Value Addition in My Smart Water Gate FYP

- Uses intelligent AI models to predict moisture requirements instead of relying solely on current water level.

- Supports proactive irrigation decisions, integrating real-time soil moisture + weather forecasts.

- Enables closed-loop automation where predicted needs directly trigger motorized gate control, removing human dependency.

- Allows for dynamic, crop-sensitive scheduling, adapting to field variability and optimizing both yield and water use.

- Maintains the paper’s practical strengths (web/SIM control) while extending functionality to intelligent, fully autonomous operation.

## Research Paper 16 - Design and Control System of Sluice Gate With Web-Based Information

Original Research Paper Link -

https://www.researchgate.net/publication/354999767_Autogation_An_Alternate_Wetting_and_Drying-Based_Automatic_Irrigation_and_Paddy_Water_Level_Control_System_through_Internet_of_Things

My Google Drive Link – https://drive.google.com/file/d/1rSFFPjeLrgbYDUby27PAUW6DvfGTr0-g/view?usp=drive_link

### Summary

- Authors: Tolentino et al.

- Published In: AGRIVITA Journal of Agricultural Science, 2021

- Focus:

  - Developed an IoT-based automated irrigation system using the Alternate Wetting and Drying (AWD) method to optimize water use in paddy fields.

  - Utilizes ultrasonic water level sensors, Arduino Mega, stepper motor-driven delivery gates, and Zigbee-based wireless transmission.

  - Aims to conserve water by maintaining the paddy water level between 100 mm below ground (dry threshold) and 150 mm above ground (wet threshold).

- System Features:

  - Real-time water level sensing using a buried PVC-encased ultrasonic sensor.

  - Web dashboard provides monitoring of water levels, irrigation status, and gate state (open/close).

  - Uses Zigbee + Wi-Fi + solar power for rural deployment and wireless data transmission.

  - Open-source microcontrollers control a counterweight-designed delivery gate.

  - Field-tested in Pampanga, Philippines, saving ~20% water over 3 weeks on a 2-hectare rice field.

- Conclusion:

  - The system significantly improves irrigation efficiency using a fixed AWD rule.

  - However, it lacks predictive logic, weather integration, or AI-based adaptive control.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Decision Strategy | Uses fixed AWD logic (100 mm below & 150 mm above) with reactive gate control | Predicts future irrigation needs using soil moisture forecasting & weather prediction | No adaptive or learning-based decision-making model |
| Rain Forecast Integration | No rain forecast API or prediction model used | Includes weather API or ML-based rainfall forecasting | Cannot avoid irrigation before rainfall; water may be wasted |
| Crop-Specific Scheduling | Fixed cycle based on water level only | Adjusts irrigation logic based on crop stage + evapotranspiration + soil data | No dynamic crop-stage adaptation |
| Automation Intelligence | Works on threshold logic with remote sensor + actuator feedback | Uses AI/ML models to control water release based on predicted needs | Lacks predictive intelligence, adaptive scheduling, or optimization |
| Crop Relevance | General use | Designed specifically for paddy fields with adaptive modes | Not crop-specific or stage-specific |

### Value Addition in My Smart Water Gate FYP

- Integrates ML models for predictive soil moisture estimation, making irrigation smarter and proactive.

- Includes weather-based decision-making, reducing unnecessary water use when rainfall is expected.

- Enhances AWD logic with adaptive scheduling, dynamically adjusting to field, weather, and crop conditions.

- Delivers a fully automated control loop from sensors → prediction → gate control, removing human dependency.

- Supports field-wide, scalable deployments using modular, low-cost components with real-time cloud dashboards.

- Enabling crop-specific water logic tailored to paddy cultivation

## Research Paper 17 - Water Management and Hydrological Characteristics of Paddy Fields

Original Research Paper Link -

https://www.researchgate.net/publication/381828405_Water_Management_and_Hydrological_Characteristics_of_Paddy-Rice_Fields_under_Alternate_Wetting_and_Drying_Irrigation_Practice_as_Climate_Smart_Practice_A_Review

My Google Drive Link – https://drive.google.com/file/d/10tW4zIQqhzTXwzyUqULRQcTCEfiEckaZ/view?usp=drive_link

### Summary

- Authors: Aziz, Yaacob, Hussain

- Published In: IOP Conference Series: Earth and Environmental Science, 2021

- Focus:

  - Evaluates the effectiveness of the Alternate Wetting and Drying (AWD) method for water management in paddy cultivation.

  - Compares AWD with continuous flooding (CF) using two experimental blocks in a 1200 m² field.

- Methodology:

  - AWD: Water allowed to drop to 15 cm below the surface before re-irrigation.

  - CF: Constant water level maintained throughout the season.

  - Parameters monitored:

    - Water table depth

    - Soil moisture at 15 cm and 30 cm

    - Water input and rainfall

  - Data collected using perforated water tubes, moisture sensors, and manual water logging.

- Findings:

  - AWD reduced water usage by ~29.2% compared to CF.

  - Soil moisture under AWD was stable and did not drop below safe crop levels.

  - Water table in AWD fluctuated between -5 cm to -20 cm, versus 0 cm in CF.

  - No adverse effects on plant growth reported.

- Conclusion:

  - AWD is an effective water-saving method.

  - However, the system is entirely manual — no automation, sensors, or predictive scheduling are included.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Control Mechanism | Manual tracking and flooding; irrigation done by field staff | Automated gate actuation using real-time sensor and AI prediction | No automation or remote execution; fully manual |
| Irrigation Scheduling | Based on visual monitoring and fixed AWD threshold (15 cm below surface) | Predicts exact water need based on sensor fusion (moisture, forecast, etc.) | No forecasting or intelligent scheduling capability |
| Rainfall Prediction | Rainfall only recorded manually; not used to delay or skip irrigation | Includes weather API/rain prediction to optimize timing | Misses opportunity to conserve water during natural rainfall |
| Data Source & Feedback | Uses PVC tubes and manual sensor readings; no dynamic data loop | Live IoT-based sensors for real-time feedback and prediction | Data not used dynamically to adjust irrigation decisions |

### Value Addition in My Smart Water Gate FYP

- Automates the proven AWD technique using sensor-driven water level and moisture monitoring.

- Adds AI-powered prediction and weather integration to skip or delay irrigation when rain is expected.

- Replaces manual irrigation with microcontroller-based motorized gates, ensuring accurate and timely water release.

- Transforms field data into actionable control decisions, enabling scalable deployment across farms without human intervention.

- Enables smart, adaptive irrigation schedules, improving water conservation and reducing labor.

## Research Paper 18 - Prediction of Rice Water Requirement Using FAO-CROPWAT-8.0 Model in Dehradun, Uttarakhand, India

Original Research Paper Link -

https://www.researchgate.net/publication/340302993_Prediction_of_Rice_Water_Requirement_Using_FAO-CROPWAT-80_Model_in_Dehradun_Uttarakhand_India

My Google Drive Link –

https://drive.google.com/file/d/1N1jW-dN_B98mC-QuHPlpwmRAJ12Tz9sq/view?usp=drive_link

### Summary

- Authors: Vikash Singh & Ramesh Verma

- Published In: International Journal of Current Microbiology and Applied Sciences, 2020

- Focus:

  - Estimates water and irrigation requirements for rice in Dehradun using the FAO CROPWAT-8.0 model.

  - Utilizes crop coefficient method, FAO Penman-Monteith equation, and USDA-SCS method for rainfall effectiveness.

- Methodology:

  - Collected meteorological parameters (temperature, humidity, wind speed, solar radiation).

  - Calculated reference evapotranspiration (ETo).

  - Simulated monthly effective rainfall and crop evapotranspiration (ETcrop).

  - Determined net irrigation requirement in mm/month and MCM for the region.

- Results:

  - Peak rice water demand: June (8.9 MCM) and July (11.2 MCM).

  - Maximum ETo: May (5.28 mm/day); Minimum: January (1.43 mm/day).

  - AWD (Alternate Wetting and Drying) not considered; purely planning-based.

- Conclusion:

  - Model helps design region-specific irrigation schedules to improve water use efficiency.

  - However, the system is manual, simulation-based, and not linked to any real-time sensor or automated control logic.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Irrigation Planning | Uses monthly averages via CROPWAT simulation | Predicts real-time irrigation need using sensors + ML | Lacks live adaptability; offline estimation only |
| Forecasting and Responsiveness | Static evapotranspiration-based schedule | Rain forecast + soil sensor fusion used to skip/delay irrigation | Cannot adapt to real-time weather changes |
| Execution & Automation | Planning tool for humans to manually irrigate | Microcontroller-based gate control automates irrigation | No automation; only gives irrigation estimates |
| Data Source | Historical climate data + crop coefficients | Real-time IoT sensors and weather API input | No real-time field data or adaptive decision-making |
| Forecast Use | Uses historical climate data only | Includes upcoming weather forecasts for intelligent timing | No forward-looking decisions |

### Value Addition in My Smart Water Gate FYP

- Transforms static, seasonal simulations into real-time predictive irrigation management.

- Replaces manual irrigation scheduling with automated gate actuation using microcontrollers and live sensor input.

- Enhances water use efficiency by adjusting in real-time to rainfall forecasts, water tank level and soil moisture predictions.

- Deploys a scalable, sensor-integrated field system, applicable beyond theoretical modeling.

- Considers rainfall, temperature, soil type, tank level to optimize gate timing

## Research Paper 19 - IoT-based Smart Irrigation System using Machine Learning for Agricultural Water Management

Original Research Paper Link -

https://arxiv.org/abs/2301.00899?utm_source=chatgpt.com

My Google Drive Link –https://drive.google.com/file/d/19KdhzNfy0dCoY0UtdGuNUHA3kE4b2yyM/view?usp=drive_link

### Summary

- Authors: Yuji Saikai, Gregory Peake, Karine Chenu

- Published In: arXiv (2023)

- Focus:

  - Proposes a Deep Reinforcement Learning (DRL)-based system for optimizing irrigation scheduling in wheat farming.

  - The model learns to make irrigation decisions daily using high-dimensional sensor feedback and is trained entirely within the APSIM-Wheat crop simulator environment.

- Machine Learning Model Description:

  - The irrigation problem is modeled as a Markov Decision Process (MDP).

  - The agent (policy network) receives a 9-dimensional state vector as input, which includes:

    - Crop stage

    - Leaf area index (LAI)

    - Extractable soil water from 5 different soil layers

    - Cumulative rainfall

    - Cumulative past irrigation

  - The action space is discrete, allowing the agent to choose one of five irrigation levels daily (0 mm, 10 mm, 20 mm, 30 mm, or 40 mm).

  - The reward function maximizes profit:

Reward=Grain Revenue−Irrigation Cost\text{Reward} = \text{Grain Revenue} - \text{Irrigation Cost}Reward=Grain Revenue−Irrigation Cost

  - The DRL algorithm used is REINFORCE, a policy gradient method:

    - A fully connected neural network (1.4 million parameters) serves as the policy model.

    - The network outputs a probability distribution over possible irrigation actions.

    - The model is trained over 20,000 episodes, each simulating a full crop season.

  - Evaluation was conducted on 10 years of unseen weather data (2011–2020).

    - The DRL policy consistently outperformed the conventional rule-based irrigation strategy.

    - In some years, it yielded up to 17% more profit due to better water-use efficiency and yield outcomes.

- Conclusion:

  - The model demonstrates that deep reinforcement learning can learn adaptive irrigation strategies directly from simulation data.

  - However, the system operates entirely in a virtual environment and is not yet connected to real-world sensors or automation hardware.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Application Domain | Simulated wheat fields in Australia using APSIM | Real-world paddy cultivation with dynamic water gates | Not applied to real-time gate control or water release in field irrigation |
| Execution Mechanism | DRL produces irrigation recommendations in mm/day; no actuator system | Direct control of sluice gates based on predicted water needs | Does not link predictions to physical automation infrastructure |
| Sensor Usage | Uses virtual sensors in a simulator (e.g., extractable soil water, LAI) | Uses physical soil and water level sensors in field | Relies on simulator inputs; lacks hardware-level integration |
| Forecast Integration | Uses historical weather, no real-time API forecast in loop | Integrates real-time weather forecasts/rainfall predictions | Not dynamic to real-world sudden rain or dry periods |
| System Type | Optimization and decision-support system only | End-to-end autonomous control system | No deployment framework for full field automation |

### Value Addition in My Smart Water Gate FYP

- Bridges the gap between AI prediction and real-time actuation by automating gate control based on soil moisture forecasts.

- Extends DRL-driven insights from simulation into a real-world operational context, with water level sensors, gate motors, and weather APIs.

- Implements closed-loop control from sensing → prediction → gate actuation, enabling intelligent and adaptive irrigation scheduling.

- Designed for paddy cultivation, addressing a different crop and hydrological requirement than simulated dryland wheat farming.

- Supports dynamic scheduling in response to actual and forecasted rainfall, reducing water waste and human intervention.

## Research Paper 20 - Advanced Prediction of Crop Water Requirements Using Machine Learning Models

Original Research Paper Link -

https://eprint.ijisrt.org/id/eprint/357/?utm_source=chatgpt.com

My Google Drive Link – https://drive.google.com/file/d/1MmkRJ7Df9nmYuVZ8EAqNpeoPo0o4Nk1a/view?usp=drive_link

### Summary

- Authors: Manikumari Nagappan, Swetha Jayamurugan, Aarthika Kudiarasumani

- Published In: International Journal of Innovative Science and Research Technology, 2025

- Focus:

  - Develops machine learning models to predict crop water requirements (CWR) by estimating Evapotranspiration (ETo) using meteorological parameters.

  - Emphasizes optimizing irrigation efficiency and promoting sustainable agriculture.

- Machine Learning Models Used:

  - Long Short-Term Memory (LSTM):

    - A deep learning model ideal for time-series prediction.

    - Captures long-term temporal dependencies in weather data.

    - LSTM outperformed other models in accuracy: RMSE = 0.45 mm/day, R² = 0.92.

  - Support Vector Machine (SVM):

    - Effective for non-linear regression using RBF kernels.

    - Slightly less accurate: RMSE = 0.58 mm/day, R² = 0.89.

  - Other models like ANN, RF, and GBM were tested and ranked based on feature importance and performance.

- Data Sources:

  - Meteorological data from IMD, NASA POWER, FAO CLIMWAT, and local stations.

  - Parameters: Temperature, humidity, solar radiation, rainfall, wind speed, sunshine hours.

- Preprocessing Techniques:

  - Normalization (Min-Max/Z-score), KNN imputation for missing values, PCA for dimensionality reduction.

  - Feature importance determined using Random Forest and GBM rankings.

- Deployment:

  - A Flask-based web application allows users (e.g., farmers) to input weather data and receive real-time CWR predictions.

- Use Case:

  - For maize in Tamil Nadu with given conditions, the system predicted a CWR of 3.69 mm/day.

- Conclusion:

  - ML models, particularly LSTM, can significantly improve CWR prediction accuracy.

  - Future work includes integrating IoT sensors, hybrid models, explainable AI, and mobile/web applications for widespread adoption.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Execution & Automation | Web app provides CWR prediction; manual execution required | Fully automates gate control via microcontroller | No real-time gate actuation or automated irrigation |
| Real-Time Integration | Relies on user-entered weather data | Uses live sensor + forecast API for autonomous decision-making | No sensor-to-actuator pipeline for closed-loop irrigation |
| Irrigation Scheduling | Provides predicted ETo or CWR; irrigation left to user | Calculates and executes required irrigation volume | No control logic tied to predicted values |
| Crop Adaptability | Focused on maize; fixed Kc values | Dynamic adaptation based on crop type, soil, weather, and growth stage | Static coefficients and fixed estimation logic limit flexibility |
| Crop Scope | General (tested for maize) | Tailored for paddy cultivation | No paddy-specific modeling |

### Value Addition in My Smart Water Gate FYP

- Closes the loop from prediction to actuation by linking ML-based CWR estimates with real-time gate control.

- Uses sensor fusion (soil, tank, weather) and ML predictions to automate irrigation volume and timing.

- Enhances model usability by removing reliance on manual web inputs; replaces them with live environmental sensing.

- Offers scalable field deployment, combining accurate ML prediction with autonomous motor control.

- Supports forecast-driven, field-specific, crop-sensitive irrigation planning, leading to optimized water use and yield.

- Tailored for paddy fields, not just any crop.

## Research Paper 21 - Proper Predictions of the Water Fate in Agricultural Lands: Indispensable Condition for Better Crop Water Requirements Estimates

Original Research Paper Link -

https://www.sciencedirect.com/science/article/pii/S2090447921000678

My Google Drive Link – https://drive.google.com/file/d/1ee0iXyMEcmzzPTYA2x6Zf1yXRyaQ2LN_/view?usp=drive_link

### Summary

- Authors: Ahmed Awad, Luo Wan, Mustafa El-Rawy, Mohamed Galal Eltarabily

- Published In: Ain Shams Engineering Journal, 2021

- Focus:

  - Investigates how the accuracy of predicting water fate in farmland affects Crop Water Requirement (CWR) estimation.

  - Compares empirical vs. hydrology-based models using:

    - CROPWAT: Empirical model using fixed formulas for effective rainfall.

    - DRAINMOD: Hydrology simulator that models infiltration, runoff, and drainage flux under real field conditions.

- Proposed Method:

  - Introduces a new approach using DRAINMOD's drainage simulations to determine CWR:

    - If daily drainage flux > 0, the soil is above field capacity → No irrigation needed.

    - If no flux, CWR = ET − infiltration.

    - In controlled drainage conditions, the system also considers water table vs. field capacity.

- Testing & Results:

  - Site: 12 ha wheat land in Yangtze River, China.

  - CROPWAT estimated CWR as 30,840 m³; the new DRAINMOD approach gave 12,750 m³ (a ~59% reduction).

  - Field scenarios tested: varying drainage practices, pipe spacing, and hydraulic conductivity.

  - Changes in water fate caused CWR variation up to 79%.

  - Empirical models (e.g., CROPWAT) could not adapt to such field changes.

- Conclusion:

  - Accurate hydrological modeling is crucial for precise irrigation planning.

  - The proposed method improves CWR estimation and supports sustainable water use.

Identified Technological Gaps

| Area | Given Paper | My Proposed Smart System | Technological Gap |
| --- | --- | --- | --- |
| Execution Method | Simulation-based offline approach using DRAINMOD | Fully automated system with real-time prediction + gate control | No real-time irrigation execution |
| Sensor & Forecast Integration | Uses weather data only for simulation; no live updates or API integration | Uses real-time sensors + forecast data (rain, evapotranspiration) | Not adaptive to real-time changes in field conditions |
| Predictive Logic | Relies on simulation logic (drainage = no irrigation) | Uses ML models to predict exact future moisture requirement | No AI-based forecasting or data-driven optimization |
| Automation Level | Manual modeling and interpretation | Autonomous decision-making linked to motorized gate actuation | No actuation or control layer |
| Target Crop & Field | Focused on wheat in drained temperate fields | Tailored for paddy cultivation, where continuous water management is critical | Does not address paddy-specific needs or standing water management |

### Value Addition in My Smart Water Gate FYP

- Converts accurate water fate modeling into a real-time, ML-driven decision-making system that triggers irrigation.

- Integrates live soil moisture, rainfall, and water level sensors to make dynamic, location-specific predictions.

- Adds an execution layer via microcontroller-operated sluice gates, closing the loop from prediction to action.

- Makes the system adaptive and scalable for different field scenarios and crop types without requiring full hydrological simulations.

- Bridges simulation and execution, ensuring water is delivered when and where needed based on data-driven insight.

- Tailored for paddy fields, not just any crop.

## Research Paper 22 - IoT-Driven Smart Irrigation for Paddy Cultivation using BCO-ResNet: A Deep Learning Approach

Original Research Paper Link -

https://ieeexplore.ieee.org/document/10940266

My Google Drive Link – https://drive.google.com/file/d/16zmwoWzSgKKzRkLDX0617G-IWV7o0MNc/view?usp=drive_link

## Paper Summary

Title: IoT-Driven Smart Irrigation for Paddy Cultivation using BCO-ResNet: A Deep Learning Approach
Authors: R. Anbarasu and V. Parthipan
Published in: Proceedings of the International Conference on Electronics and Renewable Systems (ICEARS 2025), IEEE

### Objective

The study introduces a hybrid Bee Colony Optimisation–ResNet (BCO-ResNet) model to improve irrigation management in paddy fields using IoT-based real-time data. The aim is to reduce water waste, enhance crop yield, and automate irrigation decision-making.

### System Overview

The proposed IoT-BCO-ResNet system integrates:

- IoT Sensors: Measure soil moisture, temperature, humidity, and water level.

- BCO (Bee Colony Optimisation): Performs feature selection and hyperparameter tuning to select the most relevant features (e.g., soil moisture, temperature).

- ResNet (Residual Neural Network): Deep learning model that predicts irrigation demand based on optimised input features.

- Automated Water Distribution: The output controls water flow using pumps and valves in real time.

### Workflow (Step-by-Step)

- Data Collection: IoT sensors collect multi-parameter data every 15 minutes and send it to the cloud.

- Pre-processing: Cleansing, normalisation, and outlier removal.

- Feature Selection: BCO identifies key environmental parameters.

- Model Training: ResNet learns irrigation demand patterns using historical data.

- Real-Time Prediction: The trained model predicts the optimal irrigation amount dynamically.

- Actuation: Automated pumps distribute water as required.

- Feedback Loop: System continuously monitors and refines control performance.

### Dataset and Experimental Setup

- Data collected from 10 paddy fields over 6 months.

- Implementation: Python (TensorFlow, Scikit-learn)

- Hardware: Intel i9 CPU, NVIDIA RTX 3080 GPU.

- Comparative Models: Rule-based system and SVM.

### Results

| Metric | Rule-Based | SVM | BCO-ResNet (Proposed) |
| --- | --- | --- | --- |
| Accuracy | 78.2% | 80.5% | 91.3% |
| Precision | 69.5% | 76.0% | 88.2% |
| Recall | 74.0% | 77.5% | 89.3% |
| F1-Score | 71.6% | 77.8% | 88.3% |

Outcomes:

- Water use efficiency improved by 38%.

- Crop yield increased by 22%.

- Decision-making time reduced by 18%.

## Technological Contributions

- Hybrid BCO-ResNet Architecture: Combines deep residual learning with swarm intelligence (BCO) for improved prediction accuracy.

- IoT Integration: Real-time multi-sensor monitoring for adaptive irrigation control.

- Feature Optimisation: Automatic selection of the most influential features via BCO.

- Closed-Loop Automation: Dynamic control based on predicted irrigation demand.

Identified Technological Gaps (Compared to Your Project)

| Aspect | BCO-ResNet Paper | Your Smart Water Gate Project | Identified Gaps |
| --- | --- | --- | --- |
| System Architecture | Uses cloud-based IoT sensors for prediction and actuation. | Employs a local edge-based CPS (Raspberry Pi with on-site control). | The paper relies on high-compute GPU systems; lacks edge intelligence or offline resilience. |
| Data Scope | Six-month dataset from 10 paddy fields. | Integrates real-time rainfall, soil moisture, and tank-level sensors for local irrigation control. | Missing hydrological parameters and rainfall forecasting integration. |
| Forecasting Ability | Predicts immediate irrigation needs (reactive). | Aims to include daily forecasting of water requirements using ML. | No temporal forecasting capability (short-term only). |
| Control Layer | Uses automated valves but focuses mainly on prediction. | Integrates predictive + control automation (Smart Gate with actuation logic). | Control is not deeply coupled with ML in the BCO-ResNet model. |
| Scalability | Requires GPU-based processing; not field-deployable for small farms. | Designed for low-cost IoT and edge hardware (Raspberry Pi). | Computationally expensive; no resource-efficient adaptation for small-scale farmers. |
| Model Transparency | Deep learning (ResNet) model is a black-box approach. | Uses interpretable ML and sensor-driven threshold prediction. | Explainability gap – limited model interpretability for farmers. |
| Validation | Simulated environment only (Python). | Field-deployable prototype for real-world paddy irrigation. | No hardware integration testing reported. |

## Conclusion

After a comprehensive review of 21 leading research papers in the areas of soil moisture prediction, crop water requirement estimation, and smart water gate control systems, it is evident that none of the existing works offer a complete, real-time, predictive, and autonomous solution tailored for paddy irrigation.

While various papers demonstrate advancements in:

- Soil moisture prediction using ML/ANN/LSTM,

- Water management models like FAO-CROPWAT, DRAINMOD, and APSIM,

- IoT-based gate control using sensors, fuzzy logic, PID, or GSM,

each study falls short in one or more of the following aspects:

- No system closes the loop from prediction to actuation (i.e., from moisture/rain prediction to actual gate movement).

- Limited or no integration of AI-driven irrigation scheduling (most use rule-based, threshold-based, or simulation-only logic).

- Absence of real-time decision-making based on dynamic field data (e.g., tank water level, soil moisture trends, rainfall forecasts).

- Most are tailored for general or dryland crops, and fail to address the unique hydrological demands of paddy cultivation.

### Innovation of My Proposed FYP

My proposed Smart Water Gate System introduces a novel, integrated solution by:

- Combining real-time environmental sensing(e.g.- water level , soil moisture) , weather forecasting, and AI-powered predictive modeling.

- Automating motorized gate control based on forecasted soil moisture needs — a fully closed-loop system.

- Specifically optimizing for paddy fields, where water needs vary significantly by phase and waterlogging must be precisely managed.

- Supporting scalability, modular deployment, and data-driven adaptability over time.

| Area | Paper | Key Focus / Approach | Gap Identified |
| --- | --- | --- | --- |
| Soil Moisture Prediction | Estimation of Soil Moisture in Paddy Field using ANN | Used artificial neural networks to estimate soil moisture from field data. | Did not integrate rainfall/temperature forecasts; no real-time deployment. |
| Soil & Climate Factors | Generating Soil Salinity, Moisture, and pH from Remote Sensing | Used remote sensing data (satellite) to derive soil conditions. | Relies on external imagery; lacks real-time sensor integration. |
| Hybrid Models | Hybrid Machine Learning for Soil Moisture Forecasting | Combined ML techniques for soil moisture prediction. | Focused on algorithm accuracy, not real-world automated irrigation. |
| Deep Learning | Deep Learning for Proximal Soil Sensor Development | Applied DL for improved soil sensor calibration. | Focused on sensor optimization, not irrigation control. |
| Remote Sensing | Estimation of Root Zone Moisture from Ground & Remote Sensing | Used satellite + ground truth for moisture estimates. | Not field-deployable; limited to data-rich contexts. |
| IoT / Automation | Automatic Gate Controlling System for Sustainable Agriculture (Sri Lanka) | IoT-based automatic gate controller with GSM alerts. | Automated gates but lacked predictive analytics; reactive only. |
| IoT / Gate Control | Automated Water-Gate Controlling System for Paddy Fields | Simple Arduino-controlled water gate. | No ML prediction; fixed thresholds only. |
| IoT + Web | Design and Control System of Sluice Gate with Web Interface | Gate system controlled via web dashboard. | Requires farmer intervention; not intelligent or predictive. |
| Smart System | Gate Guard: Streamlining Agriculture with Smart Water Gate | Introduced concept of smart irrigation gates. | More conceptual; lacked ML-based soil moisture prediction. |
| Cost-Effective Models | Low-Cost Approach for Soil Moisture Prediction | Used low-cost sensors with statistical models. | Accuracy limited; no weather data integration. |
| Optimized Algorithms | Optimized Soil Moisture Prediction with ML | Focused on improving predictive algorithm performance. | Algorithmic; no real-world hardware validation. |
| Hydrological Models | Water Management and Hydrological Characteristics | Hydrological simulations for water usage. | Simulation-based, not sensor-driven. |
| Climate-Aware Models | Proper Predictions of the Water Fate in Agricultural Lands | Compared CROPWAT with hydrological model. | Simulation-only, no IoT deployment. |
| Sustainable Farming | Automatic Gate System for Agriculture Water Management in Sri Lanka (Review) | Review of gate systems for sustainability. | Did not present an AI-driven, field-ready solution. |
| Multi-Sensor ML | Multi-Layer High Resolution Soil Moisture Estimation | Combined multiple sensing layers + ML. | Focused on accuracy, not automation. |
| Hybrid Forecasting | Soil Moisture Forecasting by Hybrid ML | Used hybrid machine learning techniques. | No link to irrigation hardware. |
| Remote AI | AI for Soil Sensing & Irrigation | Discussed DL/ML potential for precision irrigation. | More theoretical than practical implementation. |
| IoT Cloud | Sluice Gate with IoT Cloud Control | Designed cloud-based sluice monitoring. | Needs internet access; no offline intelligence. |
| Local IoT | Smart Gate Local Control in Sri Lanka | Embedded microcontrollers for water management. | No predictive model; basic automation only. |
| Agricultural Software | Survey of Soil Moisture Prediction Software/Hardware | Compared software frameworks and sensors. | Lacked integrated prediction + automation. |
| Smart Integration | Recent Advances in Smart Irrigation | Survey of global approaches. | Identified need for unified, predictive systems. |