# Soil moisture sensor connection

| ADS1115 Pin | → | Raspberry Pi Pin | GPIO/Function |
| --- | --- | --- | --- |
| VDD | → | Pin 1 | 3.3V Power |
| GND | → | Pin 9 | Ground |
| SCL | → | Pin 5 | GPIO 3 (SCL) |
| SDA | → | Pin 3 | GPIO 2 (SDA) |
| ADDR | → | ADS1115 GND | (for I2C address 0x48) |

| Sensor Pin | → | Connection |
| --- | --- | --- |
| VCC (Red) | → | Pin 1 (3.3V) - shared with ADS1115 |
| GND (Black) | → | Pin 9 (GND) - shared with ADS1115 |
| AOUT (Signal) | → | ADS1115 A0 pin |

Raspberry Pi 4 Model B:

├─ Pin 1 (3.3V) ──┬─> ADS1115 VDD

│                 └─> Sensor VCC

├─ Pin 3 (SDA) ────> ADS1115 SDA

├─ Pin 5 (SCL) ────> ADS1115 SCL

└─ Pin 9 (GND) ──┬─> ADS1115 GND

└─> Sensor GND

ADS1115:

├─ ADDR ──> GND (on ADS1115)

└─ A0 ────> Sensor AOUT

# Rain Detection sensor connection

| YL-83 Pin | → | Raspberry Pi Pin |
| --- | --- | --- |
| VCC | → | Pin 1 (3.3V) or Pin 2 (5V) |
| GND | → | Pin 9 (GND) |
| DO | → | Pin 11 (GPIO 17) |
| AO | → | Not connected |

# HC-SR04 Ultrasonic Sensor Pin Connections

| HC-SR04 Pin | → | Raspberry Pi Pin | GPIO/Function |
| --- | --- | --- | --- |
| VCC | → | Pin 2 | 5V Power |
| GND | → | Pin 6 | Ground |
| TRIG | → | Pin 16 | GPIO 23 (Trigger) |
| ECHO | → | Via Voltage Divider → Pin 18 | GPIO 24 (Echo) |

Step-by-step:

- HC-SR04 ECHO → One end of 1kΩ resistor

- Other end of 1kΩ → Connect to TWO things:

  - Raspberry Pi Pin 18 (GPIO 24)

  - One end of 2kΩ resistor

- Other end of 2kΩ → Raspberry Pi GND (Pin 6)